Type Certificate Data Sheet

Type certificate data sheet (TCDS); contain information relevant to the certification of particular aircraft. TCDS contain information that is useful, not only at the time the aircraft undergoes certification, but as an ongoing resource for the life of an aircraft. Any time a Certificate of Airworthiness requires renewal or re-issue, aircraft configuration or aircraft performance information is required or some particular limitations are being considered, the TCDS may provide crucial information. Following are examples of what might be found on the data sheets, although the information does vary from aircraft to aircraft. Engines and propellers that can be installed and their limitations

Fuels and oils that are approved for use in the engine, Airspeed limitations

Weight and balance limits, including the center of gravity range and the datum

- The means for leveling the aircraft

- Fuel and oil capacities and amounts that is unusable

- Control surface movements operating ceiling

- The certification basis for the aircraft

- Equivalent safety items

- Special conditions

- The flight manual that is applicable to particular models

- Placarding requirements

- Configuration variations (e.g. whether the aircraft may operate without a prop spinner)

- Variations between aircraft models

All data listed in the TCDS is approved data by the Administrator. Thereby an FAA Form 337 is not required. However, an entry is still required in the appropriate logbook.